RISC-V SoC on FPGA

Complete Source of Truth

Jayden's Personal Reference  |  Tang Nano 20K  |  RV32I

What This Document Is This is a living source of truth for the end-to-end RISC-V SoC project. It covers every concept, decision, and checklist you need from pre-coding architecture work through to a blinking LED on real hardware. Keep this open alongside your project files. Scope: RV32I base ISA ¦ 3-stage pipeline ¦ Wishbone B4 bus ¦ UART, GPIO, PWM, Timer ¦ Tang Nano 20K (Gowin GW2AR-18)

# 1. Pre-Coding Work (Do This First)

Before writing a single line of Verilog or C, complete all five of the following artefacts. Each one is an input into the next. Time spent here saves several days of debugging later.

## 1.1 Memory Map (First Artefact to Create)

Lock down the complete memory map before anything else. This is the contract between your hardware (Verilog) and firmware (C). Changing it mid-project means editing both the bus interconnect and the HAL simultaneously.

| Address Range | Peripheral | Registers |
| --- | --- | --- |
| 0x00000000 - 0x00007FFF | Block RAM | Instruction + data memory (32 KB) |
| 0x10000000 | UART | TX_DATA: write byte to transmit |
| 0x10000004 | UART | RX_DATA: read received byte |
| 0x10000008 | UART | STATUS: bit 0 = TX busy, bit 1 = RX valid |
| 0x1000000C | UART | CONTROL: bit 0 = RX interrupt enable |
| 0x10001000 | GPIO | DIR: 1 per pin, 1 = output |
| 0x10001004 | GPIO | OUT: write output values |
| 0x10001008 | GPIO | IN: read input values (read-only) |
| 0x10002000 | PWM | PERIOD: counter top value |
| 0x10002004 | PWM | DUTY: compare value |
| 0x10002008 | PWM | CONTROL: bit 0 = enable |
| 0x10003000 | Timer | COUNT: current counter value (read-only) |
| 0x10003004 | Timer | COMPARE: value that triggers interrupt |
| 0x10003008 | Timer | CONTROL: bit 0 = enable, bit 1 = interrupt enable |
| 0x1000300C | Timer | PRESCALER: clock divider value |

## 1.2 Block Diagrams

Draw three block diagrams. Every box becomes a Verilog module. Every labelled arrow becomes a port. You cannot shortcut this.

### Top-Level SoC Diagram

Show the CPU, Wishbone interconnect, BRAM, and all four peripherals as boxes. Annotate every connection with its signal names and bit widths (e.g. wb_addr[31:0], wb_dat[31:0], wb_cyc, wb_stb, wb_ack, wb_we, wb_sel[3:0]).

### CPU Internal Pipeline Diagram

Show the three stages (Fetch, Execute, Memory+Writeback) as labelled boxes with the pipeline registers between them. Write inside each pipeline register exactly which signals it holds: PC value, instruction word, decoded control signals, ALU result, register addresses.

### Wishbone Interconnect Diagram

Show the address decode logic and the read-data mux. Write the actual address ranges on the diagram. Show which ack signal feeds back from which peripheral.

## 1.3 Timing Diagrams and Clock Cycle Estimation

This is the most important pre-coding exercise and most people skip it. Draw cycle-by-cycle waveforms for each of the following scenarios before touching Verilog.

### Bus Transactions to Draw

- Single-cycle BRAM read: which cycle does stb assert, which cycle does ack return, which cycle does the CPU capture data

- Single-cycle BRAM write: same analysis

- UART TX register write: how many cycles between CPU writing and UART asserting ack

- UART status register read: same analysis

- Load instruction (LW) end-to-end through the 3-stage pipeline: label every signal on every cycle

### Pipeline Hazard Timing

For a back-to-back ADD then SUB where SUB depends on ADD's result, draw out the cycle diagram showing exactly where the stall cycle is inserted and which signals are frozen:

Cycle:   1     2     3     4     5     6
ADD:    [F]   [EX]  [MW]
SUB:          [F]   [--]  [EX]  [MW]   <- stall inserted
              ^frozen     ^reads written-back x1

### UART Baud Rate Calculation

Do this explicitly now so there are no surprises during peripheral implementation:

Clock frequency:     27,000,000 Hz
Target baud rate:    115,200 baud
Cycles per bit:      27,000,000 / 115,200 = 234.375  -> use 234
Error:               (234 - 234.375) / 234.375 = -0.16%  (within tolerance)

      
Receiver sampling strategy:
  Wait 117 cycles after start-bit falling edge (half period, centre of start bit)
  Then sample every 234 cycles for 8 data bits + stop bit
  Total frame: 1 start + 8 data + 1 stop = 10 bits = 2340 clock cycles

### Clock Budget Estimation

Your clock period is 1/27 MHz = 37 ns. Estimate whether your combinational logic fits. A 32-bit ripple-carry adder is roughly 15 gate delays. A 5-to-1 mux at the interconnect adds another 3-5 delays. At ~0.5 ns per delay you consume ~10 ns, leaving ~27 ns of slack. 27 MHz is a relaxed target; you should have no timing issues.

## 1.4 Module Interface List

Before coding any module, define its ports. Fill this table in full before opening your editor.

| Module | Inputs | Outputs |
| --- | --- | --- |
| rv32i_alu | a[31:0], b[31:0], op[3:0] | result[31:0], zero, negative, overflow |
| rv32i_regfile | clk, rst, rs1_addr[4:0], rs2_addr[4:0], rd_addr[4:0], rd_data[31:0], rd_we | rs1_data[31:0], rs2_data[31:0] |
| rv32i_decoder | instr[31:0] | rs1[4:0], rs2[4:0], rd[4:0], imm[31:0], alu_op[3:0], mem_read, mem_write, reg_write, branch, jump, lui, auipc |
| rv32i_core | clk, rst, instr_addr[31:0], instr_data[31:0], wb_* (master) | wb_cyc, wb_stb, wb_we, wb_addr[31:0], wb_dat_m2s[31:0], wb_sel[3:0] |
| wb_bram | clk, rst, wb_cyc, wb_stb, wb_we, wb_addr[31:0], wb_dat_i[31:0], wb_sel[3:0] | wb_dat_o[31:0], wb_ack |
| wb_uart | clk, rst, wb_stb, wb_we, wb_addr[3:0], wb_dat_i[31:0], uart_rx | wb_dat_o[31:0], wb_ack, uart_tx |
| wb_gpio | clk, rst, wb_stb, wb_we, wb_addr[3:0], wb_dat_i[31:0], gpio_in[7:0] | wb_dat_o[31:0], wb_ack, gpio_out[7:0] |
| wb_pwm | clk, rst, wb_stb, wb_we, wb_addr[3:0], wb_dat_i[31:0] | wb_dat_o[31:0], wb_ack, pwm_out |
| wb_timer | clk, rst, wb_stb, wb_we, wb_addr[3:0], wb_dat_i[31:0] | wb_dat_o[31:0], wb_ack, timer_irq |
| wb_interconnect | clk, rst, cpu_wb_* (from master), all slave wb_dat_o, wb_ack | All slave wb_stb, wb_we, wb_addr, wb_dat_i; cpu_wb_dat_s2m, cpu_wb_ack |

## 1.5 Test Plan

Write your test plan before writing a single test. Decisions made cold produce better coverage than decisions made mid-debug.

### ALU Test Cases

- All 10 operation codes with straightforward inputs

- ADD: check overflow wraps correctly (0xFFFFFFFF + 1 = 0x00000000)

- SRL vs SRA on a negative number (bit 31 = 1): SRL fills with 0, SRA fills with 1

- Shift by 0 and shift by 31

- SLT vs SLTU: -1 (0xFFFFFFFF) is less than 0 in signed, greater in unsigned

### Register File Test Cases

- Write to x0, read back x0: must always return 0

- Write to x1, simultaneous read of x1 and x2: verify correct values returned

- Write-then-read same register in consecutive cycles

### CPU Test Cases

- Every RV32I instruction with at least one positive and one edge-case test

- Negative immediates in ADDI, LW, SW, BEQ

- Branch taken and branch not taken for all 6 branch instructions

- Branch offset landing: verify PC goes to correct target, not target+4

- Data hazard with 0 instructions between dependent pair: verify stall

- Data hazard with 1 instruction between dependent pair: verify no stall needed

- JAL and JALR: verify return address written to rd, verify PC lands correctly

- LUI and AUIPC: verify upper 20 bits set, lower 12 bits zero

### UART Test Cases

- Transmit a single byte and verify bit timing at 115,200 baud

- Receive a byte: drive uart_rx from testbench and verify RX_DATA register

- Status register: TX busy while transmitting, clears after stop bit

- Back-to-back transmit: verify second byte starts after stop bit of first

### Firmware Test Cases (Simulation)

- crt0.S zeroes BSS section correctly

- UART prints 'RISC-V SoC booted!' and all characters appear in testbench monitor

- GPIO: LED on/off toggles correctly via memory-mapped writes

- Timer: interrupt flag sets when COUNT reaches COMPARE

# 2. System Architecture

## 2.1 Stack Overview

The complete system from firmware to silicon, bottom-up:

| Layer | What It Is | You Write |
| --- | --- | --- |
| C Firmware | main.c: your application logic | Yes |
| Hardware Abstraction Layer | hal.h: volatile register macros | Yes |
| RISC-V CPU Core | Fetch, Decode, Execute, Writeback in Verilog | Yes |
| Wishbone Bus | Interconnect routing CPU requests to peripherals | Yes |
| Peripherals + BRAM | UART, GPIO, PWM, Timer, Block RAM | Yes |
| FPGA Fabric | Gowin GW2AR-18 physical chip | No (provided) |

## 2.2 Module Hierarchy

Your Verilog file structure should mirror the following hierarchy exactly. One box equals one file.

soc_top.v
  rv32i_core.v
    rv32i_fetch.v          (or inline in core)
    rv32i_decoder.v
    rv32i_alu.v
    rv32i_regfile.v
    rv32i_hazard.v         (stall and flush logic)
    rv32i_pipeline_regs.v  (IF/EX and EX/MW registers)
  wb_interconnect.v
  wb_bram.v
  wb_uart.v
  wb_gpio.v
  wb_pwm.v
  wb_timer.v

## 2.3 Address Space

The Wishbone interconnect uses the top nibble of wb_addr[31:28] to select the target, then lower bits to select the register within each peripheral:

0x0xxx_xxxx  -> Block RAM   (addr[31:28] == 4'h0)
0x1000_0xxx  -> UART        (addr[15:12] == 4'h0)
0x1000_1xxx  -> GPIO        (addr[15:12] == 4'h1)
0x1000_2xxx  -> PWM         (addr[15:12] == 4'h2)
0x1000_3xxx  -> Timer       (addr[15:12] == 4'h3)

# 3. CPU Pipeline

## 3.1 Three-Stage Pipeline

Each stage is one clock cycle. After the pipeline fills (cycle 3), one instruction completes per cycle.

| Stage | Name | Operations Performed |
| --- | --- | --- |
| Stage 1 | Fetch (F) | Read 32-bit instruction from BRAM at PC address. Issue PC+4 for next fetch. |
| Stage 2 | Execute (EX) | Decode instruction fields. Read rs1 and rs2 from register file. Run ALU. Compute branch target. |
| Stage 3 | Memory + Writeback (MW) | If load/store: bus transaction via Wishbone. Write result to rd in register file. |

## 3.2 Pipeline Registers

Two sets of flip-flops sit between stages. They hold all signals that the next stage needs.

### IF/EX Pipeline Register (between Stage 1 and Stage 2)

- pc[31:0]: address of the fetched instruction

- instr[31:0]: the raw 32-bit instruction word

### EX/MW Pipeline Register (between Stage 2 and Stage 3)

- alu_result[31:0]: output of the ALU

- rs2_data[31:0]: store data (for SW/SH/SB)

- rd[4:0]: destination register address

- pc_plus4[31:0]: return address for JAL/JALR

- Control signals: reg_write, mem_read, mem_write, mem_to_reg, wb_we, wb_sel[3:0]

## 3.3 Hazard Detection

Your hazard detection unit sits in the EX stage and compares destination registers of in-flight instructions against source registers of the instruction being decoded.

// Data hazard: EX stage result not yet written back
wire data_hazard = (ex_rd != 0) && ex_reg_write &&
                   ((ex_rd == id_rs1) || (ex_rd == id_rs2));

      
// Stall effect:
//   PC does not increment (same instruction re-fetched)
//   IF/EX register does not update (same instruction re-decoded)
//   EX stage gets a NOP bubble (all control signals = 0)

      
// Branch flush: taken branch detected in EX stage
//   Instruction currently in IF stage is invalid -> replace with NOP

## 3.4 Common CPU Bugs

Watch for these before they cost you hours x0 not hardwired to zero: all writes to rd=0 must be silently discarded. All reads of rs1=0 or rs2=0 must return 0. Sign extension errors: I-type immediates are 12 bits sign-extended. B-type and S-type split the immediate across two fields and must be reassembled before sign extension. J-type bits are scrambled. Branch offset relative to wrong PC: offset is relative to the branch instruction itself, not PC+4. Pipeline hazard with load-use: a load followed immediately by an instruction that uses the loaded value needs one stall cycle. Your hazard unit must catch this. Non-blocking assignment in combinational block: always use = in always@(*), always use <= in always@(posedge clk).

# 4. Wishbone Bus

## 4.1 Signal Reference

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wb_cyc | Master -> Slave | 1 | Bus cycle active. Held high for entire transaction. |
| wb_stb | Master -> Slave | 1 | Strobe. This specific transfer is valid now. |
| wb_we | Master -> Slave | 1 | Write enable. 1 = write, 0 = read. |
| wb_addr | Master -> Slave | 32 | Address being accessed. |
| wb_dat_m2s | Master -> Slave | 32 | Write data (master to slave). |
| wb_sel | Master -> Slave | 4 | Byte select. Which bytes of the 32-bit word are active. |
| wb_dat_s2m | Slave -> Master | 32 | Read data (slave to master). |
| wb_ack | Slave -> Master | 1 | Acknowledge. Slave has completed the transfer. |
| wb_err | Slave -> Master | 1 | Error. Invalid or unsupported access. (optional) |

## 4.2 Transaction Timing

### Single-Cycle Read (e.g. UART Status Register)

Cycle:       1           2           3
clk:         |-----|     |-----|     |-----|
wb_cyc:      ______|_______________|______
wb_stb:      ______|_______________|______
wb_we:       LOW (read)
wb_addr:     0x10000008 (UART status)
wb_dat_s2m:  XXXX        0x00000002  (RX valid)
wb_ack:      LOW         HIGH        LOW

      
CPU captures wb_dat_s2m on the cycle wb_ack is HIGH.

### Single-Cycle Write (e.g. GPIO Output Register)

Cycle:       1           2
wb_cyc:      ______|____________
wb_stb:      ______|____________
wb_we:       HIGH (write)
wb_addr:     0x10001004
wb_dat_m2s:  0x000000FF  (all LEDs on)
wb_ack:      LOW         HIGH

## 4.3 Interconnect Logic

The entire interconnect is address decode logic plus a mux. There is no arbitration because there is only one master (the CPU).

wire sel_ram   = (wb_addr[31:28] == 4'h0);
wire sel_uart  = (wb_addr[31:28] == 4'h1) && (wb_addr[15:12] == 4'h0);
wire sel_gpio  = (wb_addr[31:28] == 4'h1) && (wb_addr[15:12] == 4'h1);
wire sel_pwm   = (wb_addr[31:28] == 4'h1) && (wb_addr[15:12] == 4'h2);
wire sel_timer = (wb_addr[31:28] == 4'h1) && (wb_addr[15:12] == 4'h3);

      
// Route stb to selected slave only
assign ram_stb   = wb_stb & sel_ram;
assign uart_stb  = wb_stb & sel_uart;
// ... etc

      
// Mux read data back from selected slave
assign wb_dat_s2m = sel_ram   ? ram_dat   :
                    sel_uart  ? uart_dat  :
                    sel_gpio  ? gpio_dat  :
                    sel_pwm   ? pwm_dat   :
                    sel_timer ? timer_dat : 32'h0;

      
assign wb_ack = (sel_ram & ram_ack) | (sel_uart & uart_ack) |
                (sel_gpio & gpio_ack) | (sel_pwm & pwm_ack) |
                (sel_timer & timer_ack);

# 5. RISC-V RV32I Instruction Reference

## 5.1 Register Conventions

| Register | ABI Name | Use |
| --- | --- | --- |
| x0 | zero | Hardwired zero. Writes silently discarded. MUST always read as 0. |
| x1 | ra | Return address (convention) |
| x2 | sp | Stack pointer (convention, grows downward) |
| x3 | gp | Global pointer |
| x4 | tp | Thread pointer |
| x5-x7 | t0-t2 | Temporaries |
| x8-x9 | s0-s1 | Saved registers (callee-preserved) |
| x10-x17 | a0-a7 | Function arguments and return values |
| x18-x27 | s2-s11 | Saved registers (callee-preserved) |
| x28-x31 | t3-t6 | Temporaries |

## 5.2 Instruction Formats

Every instruction is exactly 32 bits. The opcode is always bits [6:0]. The format determines where the other fields live.

R-type: [funct7:7][rs2:5][rs1:5][funct3:3][rd:5][opcode:7]
I-type: [imm[11:0]:12][rs1:5][funct3:3][rd:5][opcode:7]
S-type: [imm[11:5]:7][rs2:5][rs1:5][funct3:3][imm[4:0]:5][opcode:7]
B-type: [imm[12,10:5]:7][rs2:5][rs1:5][funct3:3][imm[4:1,11]:5][opcode:7]
U-type: [imm[31:12]:20][rd:5][opcode:7]
J-type: [imm[20,10:1,11,19:12]:20][rd:5][opcode:7]

      
Note: B-type and J-type have scrambled immediate bits.
Sign bit is ALWAYS in bit 31 across all formats (design decision).

## 5.3 Complete Instruction List

| Instruction | Type | Operation |
| --- | --- | --- |
| ADD rd, rs1, rs2 | R | rd = rs1 + rs2 |
| SUB rd, rs1, rs2 | R | rd = rs1 - rs2 |
| ADDI rd, rs1, imm | I | rd = rs1 + sign_extend(imm) |
| AND rd, rs1, rs2 | R | rd = rs1 & rs2 |
| OR rd, rs1, rs2 | R | rd = rs1 ¦ rs2 |
| XOR rd, rs1, rs2 | R | rd = rs1 ^ rs2 |
| ANDI / ORI / XORI | I | Immediate versions of AND / OR / XOR |
| SLL / SRL / SRA | R | Shift left logical / right logical / right arithmetic |
| SLLI / SRLI / SRAI | I | Immediate shift by shamt field |
| SLT rd, rs1, rs2 | R | rd = (rs1 < rs2) ? 1 : 0 (signed) |
| SLTU rd, rs1, rs2 | R | rd = (rs1 < rs2) ? 1 : 0 (unsigned) |
| SLTI / SLTIU | I | Immediate versions of SLT / SLTU |
| LW rd, off(rs1) | I | rd = Memory[rs1 + off] (32-bit load) |
| LH / LHU | I | 16-bit load, sign-extended / zero-extended |
| LB / LBU | I | 8-bit load, sign-extended / zero-extended |
| SW rs2, off(rs1) | S | Memory[rs1 + off] = rs2 (32-bit store) |
| SH / SB | S | 16-bit / 8-bit store from lower bits of rs2 |
| BEQ rs1, rs2, off | B | if (rs1 == rs2) PC += off |
| BNE rs1, rs2, off | B | if (rs1 != rs2) PC += off |
| BLT / BGE | B | Signed less-than / greater-or-equal branch |
| BLTU / BGEU | B | Unsigned versions of BLT / BGE |
| LUI rd, imm | U | rd = imm << 12 (loads upper 20 bits) |
| AUIPC rd, imm | U | rd = PC + (imm << 12) |
| JAL rd, off | J | rd = PC + 4; PC += off (jump and link) |
| JALR rd, rs1, off | I | rd = PC + 4; PC = (rs1 + off) & ~1 |

# 6. Bare-Metal Firmware

## 6.1 Boot Sequence

CPU starts at PC = 0x00000000. The first instruction must be in crt0.S.

- Power on: PC = 0x00000000

- crt0.S runs: set sp = 0x00008000 (top of 32 KB RAM)

- crt0.S zeroes .bss section using _bss_start and _bss_end from linker script

- crt0.S calls main()

- main() initialises peripherals via memory-mapped register writes

- main() enters its event loop

- If main() ever returns: jump to infinite loop (j .) to prevent undefined behaviour

## 6.2 HAL Pattern

Every peripheral register access must use volatile. Without it the compiler may cache the value and never re-read hardware.

#define REG(base, off)  (*(volatile uint32_t *)((base) + (off)))

      
#define UART_BASE       0x10000000
#define UART_TX_DATA    REG(UART_BASE, 0x00)
#define UART_RX_DATA    REG(UART_BASE, 0x04)
#define UART_STATUS     REG(UART_BASE, 0x08)
#define UART_STATUS_TX_BUSY  (1 << 0)
#define UART_STATUS_RX_VALID (1 << 1)

      
void uart_putc(char c) {
    while (UART_STATUS & UART_STATUS_TX_BUSY);  // poll until ready
    UART_TX_DATA = c;
}

## 6.3 Build Chain

| Step | Tool | Input -> Output |
| --- | --- | --- |
| Compile | riscv32-unknown-elf-gcc | main.c, hal.h, crt0.S -> firmware.elf |
| Strip | riscv32-unknown-elf-objcopy | firmware.elf -> firmware.bin |
| Convert | Python script | firmware.bin -> firmware.hex |
| Bake | Verilog $readmemh | firmware.hex -> initialised BRAM in synthesis |
| Synthesise | Gowin EDA | All .v files + firmware.hex -> bitstream |
| Program | Gowin Programmer | bitstream -> Tang Nano 20K via USB-C |

## 6.4 Linker Script Essentials

MEMORY {
    RAM (rwx) : ORIGIN = 0x00000000, LENGTH = 32K
}
SECTIONS {
    .text   : { *(.text.start) *(.text*)   } > RAM
    .rodata : { *(.rodata*)                } > RAM
    .data   : { *(.data*)                  } > RAM
    .bss    : { _bss_start = .;
                *(.bss*)
                _bss_end = .;             } > RAM
    _stack_top = ORIGIN(RAM) + LENGTH(RAM);
}

# 7. Toolchain Reference

## 7.1 Synthesis Reports — What to Check

After every Place and Route run in Gowin EDA, read these two sections of the report:

### Resource Utilisation

| Resource | Expected for Full SoC | Available (GW2AR-18) |
| --- | --- | --- |
| LUT | 3,000 to 8,000 (15-40%) | 20,736 |
| Flip-Flop | 1,000 to 3,000 (6-19%) | 15,552 |
| BRAM | 8 to 16 blocks (17-35%) | 46 |

### Timing Analysis

- Fmax should be well above 27 MHz (target). Aim for at least 50 MHz of margin.

- Slack = (clock period) - (critical path delay). Positive = passing.

- If timing fails: look at the critical path. Usually a long combinational chain. Fix by adding a pipeline register in the middle of it.

## 7.2 Constraint File (.cst)

Map every top-level Verilog signal to a physical FPGA pin. Verify against the Sipeed Tang Nano 20K schematic before programming. Wrong pin assignments can damage the board.

IO_LOC "clk"      4;   // 27 MHz crystal
IO_LOC "rst_n"    88;  // User button
IO_LOC "led[0]"   15;
IO_LOC "led[1]"   16;
IO_LOC "uart_tx"  17;
IO_LOC "uart_rx"  18;

## 7.3 Simulation Workflow

- Write cocotb test in Python

- Run: make SIM=icarus TOPLEVEL=rv32i_alu MODULE=test_alu

- If test fails: open waves.vcd in GTKWave

- Trace signal values cycle by cycle to find the divergence point

- Fix Verilog, re-run. Repeat until all tests pass.

- Only move to hardware after full simulation coverage.

# 8. Recommended Build Order

Always build bottom-up. Never instantiate a module you have not already tested in isolation.

| Phase | Module(s) | Gate to Pass Before Moving On |
| --- | --- | --- |
| 1 | rv32i_alu | All 10 operations pass cocotb tests including edge cases |
| 2 | rv32i_regfile | x0 always zero. Two simultaneous reads correct. |
| 3 | rv32i_decoder | All 6 instruction formats decode correctly. All immediate reconstructions correct. |
| 4 | rv32i_core (single-cycle first) | All 40 RV32I instructions pass riscv-tests suite in simulation. |
| 5 | Add pipeline registers | Same riscv-tests suite still passes with pipeline. |
| 6 | rv32i_hazard | Back-to-back dependent instructions produce correct results. |
| 7 | wb_bram | Read and write transactions correct. ack timing correct. |
| 8 | wb_uart | Bit timing matches 115,200 baud. Full byte TX and RX pass. |
| 9 | wb_gpio, wb_pwm, wb_timer | Each peripheral passes isolated register read/write tests. |
| 10 | wb_interconnect | Address decode correct. No overlapping acks. CPU reads correct peripheral. |
| 11 | soc_top (simulation) | Firmware boots, UART prints, GPIO toggles in simulation. |
| 12 | soc_top (hardware) | Synthesise, program Tang Nano 20K, verify over serial terminal. |

# 9. Quick Glossary

| Term | Definition |
| --- | --- |
| ALU | Arithmetic Logic Unit. The part of the CPU that performs arithmetic and logical operations. |
| BRAM | Block RAM. Dedicated memory blocks built into the FPGA fabric, much more efficient than using LUTs as memory. |
| Bitstream | The binary file that configures an FPGA. Generated by Place and Route. Loaded via USB. |
| Fmax | Maximum clock frequency at which a design meets timing. Should be well above 27 MHz. |
| HAL | Hardware Abstraction Layer. C header defining volatile macros for peripheral register access. |
| Hazard | A pipeline situation where the next instruction cannot proceed normally. Resolved by stalling or forwarding. |
| LUT | Look-Up Table. The basic configurable logic element in an FPGA. Implements any Boolean function of its inputs. |
| Memory-Mapped I/O | Accessing peripherals via normal load/store instructions to specific addresses rather than special instructions. |
| Netlist | Intermediate representation of a circuit as gates, flip-flops, and connections. Output of synthesis. |
| P&R | Place and Route. Maps a netlist onto physical FPGA resources and routes wires between them. |
| Pipeline Register | Flip-flops that hold intermediate results between pipeline stages. |
| Prescaler | A clock divider. Counts N clock cycles before producing one output tick, slowing a signal down. |
| RTL | Register Transfer Level. The abstraction level at which Verilog describes hardware: registers and logic between them. |
| RV32I | RISC-V 32-bit base integer instruction set. 40 instructions total. What you are implementing. |
| Slack | Clock period minus critical path delay. Positive = passing timing. Negative = failing timing. |
| SoC | System on Chip. A complete computer (CPU, memory, peripherals) on a single chip or FPGA. |
| Two's Complement | The standard binary representation for signed integers. Bit 31 is the sign bit. |
| Wishbone | An open-source bus protocol connecting CPU master to peripheral slaves via a simple handshake. |
| volatile | C keyword that forces every read/write to actually access the hardware register, preventing compiler optimisation. |
| x0 (zero) | RISC-V register 0. Always reads as 0. All writes are silently discarded. Must be hardwired in hardware. |

# 10. Key References

| Resource | What to Use It For |
| --- | --- |
| riscv.org/technical/specifications (Chapter 2) | The authoritative RV32I spec. Chapter 2 covers all 40 instructions and instruction formats. |
| github.com/riscv-software-src/riscv-tests | Official test suite. Pass this in simulation before touching hardware. |
| zipcpu.com | ZipCPU Wishbone tutorials. Excellent practical walkthroughs for building bus-connected peripherals. |
| github.com/YosysHQ/picorv32 | A real working RV32I core to read and understand. Write your own from scratch but use this as a reference. |
| wiki.sipeed.com/hardware/en/tang/Tang-Nano-20K | Tang Nano 20K pinout, schematic, and Gowin EDA getting-started guide. |
| docs.cocotb.org | cocotb documentation for writing Python testbenches. |

End of document. Update this file as decisions change.